Filename: Mail-Import.properties

Location: C:\Ephesoft\Application\WEB-INF\classes\META-INF\dcma-mail-import\

Purpose: Used to configure settings for importing batch instances from email (version 2022.1.01).

dcma.importMail.cronExpression=0 0/1 * ? * *

#multiple values separated by semi-colon(;)

dcma.supported.attachment.extension=doc;xls;zip

#Value indicating the threadpool size to be used by the mail import service.

dcma.mail.import.core.size=1

# Extension type for mail import body.

# 0 - HTML, default value

# 1 - TEXT

dcma.mail.import.mail_body_extension_type=0

#dcma.mail.import.max_mail_retry_count tells how many number of times try to process mail

# again in case of failure

dcma.mail.import.max_mail_retry_count=3

# mail retry interval

dcma.mail.import.mail_retry_interval=2000

#mail body download flag

#1= don't download body, download only attachments

#0= download body with attachments(default value)

dcma.mail.import.mail_body_download_flag=0

#mail processing cron expression:

dcma.importMail.mailProcessCronExpression=0 0/5 * ? * *

#max cumulative mail processing count.

dcma.mail.import.max_mail_processing_capacity=5

#To check the procedure to be followed for picking up e-mails.

#automated, for simple pickup of e-mail = 0

#timed, for clubbing of e-mails = 1

dcma.mail.import.mail_pickup_procedure=0

#flag to specify if msg file needs to be downloaded or processed

#only download msg attachment=0

#download and process msg attachments=1

dcma.mail.msg_download_and_process_flag=0

#number of mails to be downloaded per cron cycle

dcma.mail.number_of_mails_downloaded_per_cron_cycle=5

# HTML mail body processes through below softwares. Set value of dcma.mail.import.mail_conversion_tool corresponding to software

# soffice - 1

# itext - 2

# wkhtmltopdf - 3

dcma.mail.import.mail_conversion_tool=2

#These parameters are used in case of wkhtml tool.

dcma.mail.html_to_pdf_conversion_parameter=--load-error-handling ignore -q --minimum-font-size 16 --page-size A4 --zoom 1.4

#This parameter is used in case of itext tool.

#When mails are sent to Transact, the text is cut off during the conversion process.

#Tabular data are displayed in HTML tags.

#set true to resolve the above issues.

dcma.mail.itext_html_tag=false

#Switch to fetch and set headers or not.

dcma.mail.headers_switch=ON

#Headers to fetch from mail, separated by semi-colon.Example: Subject;To;From

dcma.mail.required_headers=From;To;Date;Subject;Message-ID

# Maximum number of retry attempts made to connect to mail server in case of failure.

dcma.mail.import.max_connection_retry_count=3

# Time interval in ms between each retry attempt for mail server connection.

dcma.mail.import.connection_retry_interval=500

#Is proxy configuration required for exchange server?

#false: Mail should NOT use proxy for exchange server (Default)

#true: Mail should use proxy for exchange server

dcma.mail.import.is_proxy_required=false

#proxy server ip-address

dcma.mail.import.proxy_host=127.0.0.1

#proxy server port

dcma.mail.import.proxy_port=80

#Does proxy require authentication?

dcma.mail.import.proxy_credential_required=false

#Proxy server username

dcma.mail.import.proxy_server_username=ephesoft

#Proxy server password

dcma.mail.import.proxy_server_password=ephesoft

#Proxy server domain

dcma.mail.import.proxy_server_domain=ephesoft

# IMAP and POP3 connection timeout in milliseconds

mail.util.connection.timeout=30000

# IMAP and POP3 request (I/O) timeout in milliseconds

mail.util.request.timeout=30000

# Enable/disable automatic character-set detection (from subject & body) for incoming mails

dcma.mail.import.charset.autodetect=false

# Default character-set to use when automatic detection fails for mail subject or body

dcma.mail.import.charset.default=utf-8

# Enable/disable downloading of inline attachments for incoming mails

dcma.mail.import.download_inline_attachments=true

# Remove invalid character from attachments name

dcma.mail.import.attachments_invalid_characters=[/\\\\:*?\"<>|]

#set true to remove HTML comment from email body for ITEXT conversion, By default it always be false

dcma.mail.itext_remove_html_comment=false

#Set ON/OFF to enable/disable advance feature for inline body attachment handling for incoming mails, By default it will always ON for fresh installation

dcma.mail.import.advanced_inline_body_attachment_handling=ON